Senior Capstone Project Proposal

The project is designed for a team of students working toward completion of a project, within two semesters1.

| Title | ECM Motor Interface Accessory for HVAC Control Systems |
| --- | --- |
| Sponsor | Contact person: Darin Franz, or Morgan Estill Company name: Regal Rexnord Contact info: Darin.Franz@regalrexnord.com, or Morgan.Estill@regalrexnord.com |
| Description | Many HVAC systems use 24 VAC control logic, while PSC induction motors operate directly from 120/240 VAC line voltage. When upgrading to ECM motors, customers often prefer to replace only the motor rather than the entire control system. This project will develop a physical accessory module that enables ECM motors to operate with existing line-voltage HVAC controls without modifying the control board. The student team will design, build, and test a prototype accessory that includes: - A line-voltage to 24 VAC control-signal conversion circuit using passive or active components. - Line/current sensing to detect energized inputs and provide status feedback. - Motor power (wattage) sensing for energy-use monitoring and diagnostics. - (If Possible) RPM sensing to indicate motor operating status. - Safe HVAC wiring, mounting, and enclosure design. - Optional 0–10 V to duty-cycle conversion using commercially available hardware. - Evaluation of competitive solutions to identify opportunities for improved functionality. The final deliverable will be a fully assembled and tested prototype, including electronics, enclosure, and wiring, validated for operation with ECM motors in representative HVAC applications. |
| Disciplines (ME, EE, CS, etc.) | EE CmpE |
| For ECE | Please provide details about the expected project contents. For hardware, please list the possible techniques, such as PCB design, analog circuit, signal conditioning, RF circuit, PLC, embedded system firmware, and robotic control. For software, please list possible programming languages, such as Python, C, C++, C#, Java, and Javascript.2 Hardware - Circuit design - Sensors - PCB design - Embedded systems - Prototype development and testing Software/Firmware - Embedded programming - Data collection and analysis |
| Estimated budget | $2000 |
| Technology Disclosed? If so, what? | N/A |
| Additional requirements | N/A |
| NDA or IP Assignment agreement requested? | N/A |

Technology and ECCN:

“If your project involves ‘technology’ that is either (a) not publicly available or (b) includes proprietary source code (not executable files), then it requires an ECCN.” ‘Technology,’ for this purpose, is defined as “information necessary for the development, production, use, operation, installation, maintenance, repair, overhaul or refurbishing of an item. Technology may be in any tangible form, such as written or oral communications, blueprints, drawings, photographs, plans, diagrams, models, formulae, tables, engineering designs and specifications, computer-aided design files, manuals or documentation, electronic media or information revealed through visual inspection.”

Interactive tool to determine ECCN:

https://www.bis.doc.gov/index.php/export-control-classification-interactive-tool

NDAs and IP Assignments:

The sponsoring company typically has NDAs and IP assignment forms that it wishes to use.  Neither the NDA nor the IP assignment is an agreement with Purdue directly; these agreements are between the students and the sponsoring company.  Of course, our office can review the company-provided documents to be certain it aligns with Purdue’s standards.  Alternatively, our office has draft agreements which we could provide for the sponsor’s use.  Again, as NDAs are between the student and the sponsor, Purdue cannot be a party to or advise the sponsor or the student on the NDAs, other than to outline some basic expectations as to fairness and suitability of the NDA to a student project.

Sponsor Acknowledgements:

By way of background, Purdue University professors who have senior capstone class projects involving outside sponsor companies notify our office so that we can prepare an acknowledgement form for the sponsoring company’s completion. This is not a contract but an acknowledgement form signed by sponsoring companies which lays out Purdue’s guidelines regarding class projects and outside company inputs, potential export control issues, and student intellectual property. Some sponsoring companies offer a monetary donation to the project, but that is not a requirement.